Note: The Waste transaction does not change the on-hand inventory; it is for documentation purposes only. Medications must first be removed from inventory through the Take process.

#### 1. Accessing the Cabinet

- Scan ID badge or enter primary PIN code to log into MedServe.

- Select the Compartment assigned the medication to be wasted.

- Enter secondary PIN code and press the Unlock button.

#### 2. Selecting & Wasting Medication

- Choose the medication to be wasted and select the ‘Waste’ option.

- The cabinet will prompt for a witness to authorize the waste.

- The witness must scan their ID badge or enter their primary PIN code.

- Enter the quantity to be wasted and select ‘Save’.

#### 3. Adding Patient Information

##### For Non-Preloaded Patients:

- Enter the relevant patient information, which may include:

  - MRN (Patient ID)

  - Patient Name

  - Date of Birth (DOB)

  - Patient Address

##### For Preloaded Patients:

- Tap the ‘Select Patient’ button to access the patient list.

  - Note: This button is only available if there are active patients. If unavailable, no patients are active or the cabinet is not properly connected to the network.

- Use the search field to find the patient or scroll through the list.

  - Search options: Patient Name, MRN, DOB, etc.

#### 4. Completing the Waste Process

- Select the reason for waste from the dropdown menu.

- Ensure all required fields are completed.

- Press the ‘Waste’ button in the bottom right corner.

- Press the ‘Lock’ button once the transaction is complete.